ASSESMENT TOOL - 1

Computer Organisation and Architecture

Application Based Questions

- YAVANIKA S

192521258

# Question 1: Smart City Surveillance System — CPU, Memory, I/O and Bus Structures

## Understanding the Problem

Many cameras stream video continuously through I/O devices into memory, and the CPU must fetch this data for real-time threat analysis. During peak hours, congestion in the data pathway — not raw CPU power — is what causes the delay.

## How CPU, Memory and I/O Interact

- Data path: Every camera frame is transferred by an I/O controller into main memory before the CPU can process it.

- Contention: This transfer competes with the CPU's own instruction and data fetches for the same bus.

- Bottleneck: On a shared bus, only one transfer can occur at a time, so video I/O and CPU accesses queue behind each other.

## Single-Bus vs Multi-Bus Structures

- Single-Bus: One shared set of address/data/control lines links CPU, memory and I/O. Simple and low-cost, but only one transfer can happen at a time — heavy camera traffic blocks CPU-memory communication.

- Multi-Bus: Separate CPU-memory and I/O buses (joined by a bridge) let camera data move into memory while the CPU works on already-buffered frames in parallel, sharply cutting contention.

Fig 1.1 — Single-Bus vs Multi-Bus structure for CPU / Memory / I/O

## Improving the Instruction Execution Cycle

- Pipelining: Overlaps fetch, decode and execute of successive instructions so the CPU is never idle waiting on memory.

- Cache memory: Placed between CPU and main memory to cut down the number of slow main-memory accesses.

- DMA: Moves camera data directly into memory without CPU involvement for every byte, freeing the CPU for threat detection.

## Conclusion

The lag is caused by congested shared pathways, not CPU capacity. A multi-bus architecture with DMA-driven I/O transfer, pipelining, and caching together restore real-time responsiveness.

# Question 2: Real-Time Stock Trading Application — CPI and Fetch-Decode-Execute Cycle

## Understanding the Problem

Even with millions of instructions executed per second, delays appear under heavy load because CPI (Cycles Per Instruction) is high — frequent memory accesses and branch instructions make each instruction take more cycles than ideal.

## Impact on the Fetch-Decode-Execute Cycle

Fig 2.1 — Fetch-Decode-Execute cycle and where CPI is lost

- Formula view: Total execution time = Instruction count × CPI × Clock cycle time — so a high CPI directly inflates transaction latency.

- Memory waits: Every memory-access instruction forces the CPU to wait for data far slower than its internal clock, inflating CPI.

- Branch stalls: Conditional buy/sell logic can stall or misdirect the pipeline if the branch outcome isn't known in advance.

## Reducing CPI Through Architectural Enhancements

- Caching: Multi-level cache hierarchy serves frequent data/instructions quickly instead of from slow main memory.

- Branch prediction: Predicts the likely branch outcome and pre-fetches down that path, avoiding pipeline stalls.

- Pipelining: Multiple instructions progress through different stages simultaneously, raising throughput.

- Code-level tuning: Reordering code to reduce dependent memory accesses and using registers efficiently.

## Conclusion

High CPI here stems from memory-bound and branch-heavy execution. Caching, branch prediction and pipelining together bring the system closer to true real-time performance.

# Question 3: 8085 Microprocessor in Industrial Temperature Control — Addressing Modes

## Understanding the Problem

Sensor readings are occasionally processed incorrectly. The likely cause is improper use of addressing modes — the program ends up reading or writing the wrong register or memory location.

Fig 3.1 — 8085 addressing modes and where each operand comes from

## Addressing Modes and Their Influence

- Immediate: MVI A, 05H — operand is fixed in the instruction; a wrong constant means a wrong threshold value every time.

- Register: MOV A, B — data moves between registers; using the wrong register mixes up unrelated values.

- Direct: LDA 2050H — a fixed memory address; a typo or overlap with another variable silently reads stale data.

- Indirect: MOV A, M — the HL register pair points to the actual address; an un-updated or stale HL reads the wrong cell entirely.

## Likely Causes and Fix

- Root causes: An uninitialized or stale HL pointer, a mistyped direct address, or a register reused for two purposes without saving its value.

- Fix: Use clearly initialized pointers for indirect access, labelled addresses instead of raw numbers, and PUSH/POP around register reuse.

- Safety layer: Add compare-and-jump range checks after each sensor read so implausible values never reach the actuator.

## Conclusion

Addressing modes decide exactly where data comes from. Consistent, validated indirect and direct addressing — backed by range checks — removes this class of error.

# Question 4: 8086 Microprocessor — Segmented Memory Issues

## Understanding the Problem

Running multiple programs on an 8086 causes incorrect data access and stack overflow, pointing to improper handling of the segmented memory model.

Fig 4.1 — 8086 segmented memory layout (CS, DS, SS, ES)

## How Segmentation Works

- Addressing: Physical address = Segment × 10H + Offset, giving a 20-bit address from 16-bit segment and offset values.

- Four segments: CS (code) + IP, DS (variables), SS (stack) + SP, and ES (extra data) — each should ideally get its own non-overlapping base.

## How Improper Handling Causes the Problems

- Incorrect access: Overlapping segment bases between two programs let one program's data segment collide with another's code or stack, corrupting it silently.

- Stack overflow: Unmatched CALL/PUSH without corresponding RET/POP, or too small an SS allocation, lets SP grow past its segment boundary.

## Ensuring Correct Execution

- Isolation: Allocate distinct, non-overlapping segment bases per program and initialize all segment registers at start (never rely on leftovers).

- Stack discipline: Size the stack segment generously and balance every CALL/PUSH with a matching RET/POP.

- Addressing discipline: Use correct segment overrides consistently across addressing modes (direct, indexed, based) so no instruction reads the wrong segment.

## Conclusion

Segmentation isolates programs only when bases don't overlap and the stack is disciplined. Proper initialization and addressing prevent the described failures.

# Question 5: Number System Conversion and Reliable Data Processing

## Understanding the Problem

Packet data shown in hexadecimal for easy debugging is misread during conversion, leading to incorrect processing — showing how a small conversion slip can cascade into a real fault.

Fig 5.1 — Each hexadecimal digit converts to an exact 4-bit binary pattern

## Importance of Hex-Binary Conversion

- Why hex: Each hex digit maps exactly to 4 binary bits, so hex compactly represents long binary strings for human debugging while converting trivially back to binary for hardware.

- Where used: Packet headers, addresses and control codes are commonly shown this way — any misread digit changes the real binary the hardware processes.

## How Conversion Errors Affect Execution

- Operand field: A wrong operand value sends the CPU to fetch or write the wrong memory location.

- Opcode field: A wrong opcode/control field can cause an entirely different instruction to execute.

- System impact: In a communication system this shows up as misrouted packets, wrong checksums, or data written to the wrong buffer offset.

## Role of CPU Design and Benchmarking

- CPU-level checks: Parity/checksum circuits flag a byte that doesn't match its expected pattern; memory protection limits how far an errant address can corrupt.

- Benchmarking: Running standard test workloads and comparing expected vs actual output catches conversion bugs before deployment, often visible as CPI spikes from repeated faulted accesses.

## Conclusion

A single mis-converted digit changes real instructions or data. Robust CPU-level checks combined with thorough benchmarking is the practical safeguard against this class of fault.